Christopher Austin O’Connell

GTID: 902706132

ECE6390

Homework 6: Pulse Shaping

The following parameters have been used for the transmit:

- Pulse Amplitude Modulation parameters

  - Symbol rate of 1 bit per symbol:

    - 1 => 1

    - 0 => 0

  - Pulse width: 20 nanoseconds (corresponds to 50MHz)

  - Pulse repetition interval: 20 nanoseconds

    - Each bit corresponds to a different amplitude, so 100% duty cycle is applicable here to maximize throughput

  - PAM sample rate: 2^27, or 134,217,728 Hz

    - This was selected as the first power of 2 that exceeds the Nyquist Shannon theorem for a frequency higher than 50 MHz. 50MHz is the highest frequency at baseband, corresponding to +/- 50MHz that will then we upconverted to our desired FCC range of 2.4-2.5GHz (100MHz channel bandwidth).

- Amplitude Modulation parameters

  - Carrier frequency: 2.45 GHz

    - Midpoint between 2.4-2.5 GHz (selected because the baseband signal scales from +/- 50MHz

  - Modulation Index = 0.8

    - Arbitrarily selected value close to 1.0 to have a noticeable carrier envelope

Results:

Peak power outside FCC band: 62 dB less than peak value in band.

Bit rate: 50 Mbps

Note: Adhering to digital PAM chipset limitation of 4Tb of time support, the above PAM implementation could be updated to use a symbol rate of 4 bits per symbol, which would increase the overall bitrate to 200 Mbps

Graphs of results:

And here’s some downsampled graphs to show off visual clarity of the PAM and AM implementation:

Source code:

import numpy as np

import matplotlib.pyplot as plt

def pam(data):

pulse_width = 0.00000002 # 20 ns limit

Tb = 0.00000002 # PRI

bitrate = 1/Tb

print(f"bitrate: {int(bitrate)} bps")

Fs = 2**28 # 1e9 # 100MHz sample rate

Ts = 1 / Fs # Ts = period (time between each sample)

samples_per_bit = int(Tb / Ts)

samples_per_pulse = int(pulse_width / Ts)

print(f"Samples per bit: {samples_per_bit}. Samples per pulse: {samples_per_pulse}")

duration = len(data) * Tb

t = np.arange(-duration/2, duration/2, Ts) # nof_samples

# Create PAM signal (single bit per symbol)

# 0 -> 0

# 1 -> 1

pam_signal = np.zeros(len(t))

for i, bit in enumerate(data):

start = i * samples_per_bit

end = start + samples_per_pulse

pam_signal[start:end] = 1 if bit == 1 else 0

return t, pam_signal, Fs, duration

# The binary sequence to modulate

# data = np.array([1, 0, 1, 0, 1])

data = np.random.randint(2, size=500000)

# PAM

t, pam_signal, Fs, duration = pam(data)

plt.subplot(3, 2, 1)

plt.plot(t, pam_signal)

plt.title("PAM")

# AM upconvert to 2.45 GHz center frequency

carrier_amp = 1

carrier_freq = 2.45e9 # 2.45 GHz

modulation_index = 0.4 # ratio of carrier to sideband amplitude (aka carrier envelope)

carrier = carrier_amp * np.cos(2*np.pi*carrier_freq*t)

am_signal = (1 + modulation_index * pam_signal) * carrier

plt.subplot(3, 2, 2)

plt.plot(t, carrier)

plt.title(f"AM Carrier ({int(carrier_freq)} Hz)")

plt.subplot(3, 2, 3)

plt.plot(t, am_signal)

plt.title("AM modulation")

output_signal = pam_signal

# Zero pad before taking FFT

next_power_of_2 = int(np.ceil(np.log2(len(output_signal)))) + 2

target_length = 2**next_power_of_2

deficit = target_length - len(output_signal)

padded_output_signal = np.pad(output_signal, (0, deficit), mode='constant', constant_values=0)

print(f"Length of padded output: {len(padded_output_signal)}")

# FFT

output_fft = np.fft.fft(padded_output_signal)

# frequency plot points for graphing FFT and PSD

f_axis = carrier_freq + np.arange(-Fs/2, Fs/2, Fs/len(padded_output_signal))

# FFT shifted and scaled for visualization

output_fft_scaled = abs(np.fft.fftshift(output_fft))*(duration)/len(output_fft)

plt.subplot(3, 2, 4)

plt.plot(f_axis, output_fft_scaled)

plt.title(f"FFT (0-padded to {len(padded_output_signal)} samples)")

# PSD

psd = 10*np.log10(output_fft_scaled**2)

plt.subplot(3, 2, 5)

plt.plot(f_axis, psd)

plt.title("PSD")

plt.tight_layout()

plt.show()